CSC-105L DIGITAL LOGIC DESIGN

( SPRING 2026)

PROJECT REPORT

Fire and Smoke Detection Alarm System

Lab Instructor

Engr. Abdullah Bilal

Submitted by

Muhammad Awais (2025-BSCPE-136)

Sajjad Haider (2025-BSCPE-158)

Danial Mohyuddin (2025-BSCPE-156)

Electrical Engineering Department

UET Lahore (FSD Campus)

Fire and Smoke Detection Alarm System

Lab Report

Subject: Digital Logic Design     |     Project: Detection     |     Academic Year: 2026

1.  Introduction

1.1  What is the project about?

SAMOKE is a Fire and Smoke Detection Alarm System designed using digital logic gates. The system detects smoke through an MQ2 smoke sensor and elevated temperature through an LM35 temperature sensor, then activates an alarm consisting of red LEDs and a buzzer. The core logic is built around the 7432 OR Gate IC and 7404 NOT Gate IC, making it a fully hardware-based detection system without the need for any microcontroller or software.

1.2  Why is it important?

Fire and smoke are among the most dangerous and fast-spreading hazards in homes, offices, and industrial environments. Early detection can save lives and prevent major property damage. A low-cost, hardware-only alarm system like SAMOKE offers a reliable solution that does not depend on software, power-hungry processors, or complex programming — making it accessible, durable, and easy to understand for educational and practical purposes alike.

1.3  What are you trying to achieve?

The objective of this project is to build a functional fire and smoke detection circuit that:

•  Detects smoke or elevated temperature using analog sensors.

•  Activates a red LED and buzzer alarm when a hazard is detected.

•  Displays a green LED under safe and normal operating conditions.

•  Demonstrates the practical application of digital logic gates in safety systems.

2.  Methodology

2.1  List of Components Used

The following components were used to design and build the SAMOKE circuit:

| Component | Description |
| --- | --- |
| MQ2 Smoke Sensor | Detects smoke and combustible gases in the air |
| LM35 Temperature Sensor | Measures ambient temperature; outputs proportional voltage |
| 7432 OR Gate IC | Core logic gate; outputs HIGH when any sensor detects hazard |
| 7404 NOT Gate IC | Inverts OR gate output to drive normal-status green LED |
| Buzzer | Produces audible alarm when hazard is detected |
| 2 × Red LEDs | Visual alert during alarm/hazard condition |
| 2 × Green LEDs | Visual indicator of safe/normal operation |
| 4 × 220Ω Resistors | Current limiting for LEDs to prevent damage |
| Breadboard | Used for prototyping without soldering |
| Jumper Wires | Connections between components on the breadboard |
| 5V Power Supply | Powers all ICs and sensor components |

2.2  Circuit Setup

The 7432 OR Gate and 7404 NOT Gate ICs were mounted on a breadboard and powered via a 5V supply. The MQ2 smoke sensor output was connected to input A of the OR gate, and the LM35 temperature sensor output to input B. When either sensor output goes HIGH, the OR gate output becomes HIGH — activating the red LED and buzzer. This signal is simultaneously fed through the NOT gate to drive the green LED LOW (turning it off). Under normal conditions, both sensor outputs are LOW, the OR gate output stays LOW, the alarm is silent, and the green LED remains ON.

3.  Results

3.1  Outcome of the Project

The SAMOKE fire and smoke detection system performed successfully as designed. The logic gate circuit correctly processed sensor inputs and produced the expected outputs under both normal and hazard conditions. System response was immediate — as soon as smoke was introduced near the MQ2 sensor, the red LED and buzzer activated without any delay.

3.2  How the Project Works

The table below summarizes the two operating states of the SAMOKE system:

| Condition | Normal (Safe) | Hazard Detected |
| --- | --- | --- |
| Sensor Output | Both LOW | MQ2 or LM35 → HIGH |
| OR Gate Output | LOW | HIGH |
| Red LED | OFF | ON |
| Green LED | ON | OFF |
| Buzzer | Silent | Active (Alarm) |

4.  Conclusion

The SAMOKE Fire and Smoke Detection Alarm System was successfully designed and implemented using digital logic gates (7432 OR Gate and 7404 NOT Gate) paired with the MQ2 smoke sensor and LM35 temperature sensor. The system reliably detects fire-related hazards and provides both visual (LEDs) and audible (buzzer) alerts in real time.

This project confirms that fundamental digital logic can be directly applied to real-world safety problems without a microcontroller or any programming. SAMOKE is a practical, low-cost, and effective prototype for early fire and smoke detection. Future enhancements could include wireless alert integration, an LCD status display, or connection to a smart home system — making it even more reliable and scalable for real-world deployment.